CROPEDIT

A Testcard design aid for the "Radio Amateurs Testcard".

version 3.3 7-March-1993

This  program is designed to simplify the creation of  customised

testcard pictures for use in Colin Edwards excellent single board

testcard  generator (commonly known as the Cropredy TCG and  fea-

tured  in  the BATC compendium). It will produce a  binary  image

file ready for programming in an EPROM.

The  program  is written entirely in assembly language  but  uses

both BIOS and DOS function calls to ensure compatibility with all

IBM type computers. It is assembled by A86, Eric Isaacsons share-

ware assembler.

I have spent many hours working on this programs development  and

produced  about  100 personalised testcards with it.  I  make  no

guarantees about its suitability for your particular  application

but  for  most people it will save hours  of  pencil  scribblings

trying to work out which characters go where.

I will keep the latest version of Cropedit and A86 on my BBS  for

downloading any time of day or night. BBS details are given later

in this document.

Bug reports and sensible suggestions can be left on the BBS which

I check daily. I should be able to sort out problems within a day

or two.

At  present  the  program only works with 625 line  /  50Hz  CCIR

standard  video.  The hardware can easily be adapted to  work  on

other  systems including 525 line / 60 Hz NTSC. As far as I  know

it  only takes one component change and a new testcard EPROM.  If

you need a non CCIR testcard please contact me for details.

The program assumes you have a colour modulator connected to  the

TCG,  if  not you will get shades of grey  instead  of  different

colours but the layout will still be the same.

Hardware Requirements:

An IBM or compatible computer, DOS 3.0 or later,

A mouse and standard (int33) driver software,

A VGA or SVGA monitor.

Versions 3.30 thru 6.22 of MSDOS and version 3.3 of PCDOS all seem

to  work OK and although I have not tried them, it should run  on

any version from 3.0 onward.

The  mouse  must  use the standard interrupt  33  interface.  The

program  has  been tested with the Genius mouse driver  8.02,  A4

mouse  1.02 and Microsoft mouse driver 7.03. If you have  trouble

with  your  mouse it probably doesn't support VGA mode 12  and  I

would  advise  you  to update to a later version  of  its  driver

software.

Cropedit  uses VGA mode 12 which should be supported by  all  VGA

hardware.  The video mode at entry is restored when  the  program

ends.  No  problems  have been seen on Trident,  Tseng  8900  and

Paradise  cards. If you have a video bios shadow option  on  your

system I would suggest you use it. The grid drawing routines  are

rather slow and benefit from fast video bios operation.

Driving Lessons:

Cropedit splits the screen into six areas as below

/-----------------------------------------------------------\

|/-------------------------------------------\/------------\|

||                                  |        ||            ||

||                                  |        ||  Colour    ||

||                                  |        || selection  ||

||                                  |        |\------------/|

||                                  |        |/------------\|

||                                  |        ||            ||

||                                  |        ||            ||

||                                  |        ||            ||

||                                  |        ||            ||

||    Testcard editing area         |        ||   Font     ||

||                                  |        ||   editing  ||

||                                  |        ||   area     ||

||                                  |        ||            ||

||                                  |        ||            ||

||                                  |        ||            ||

||                                  |        ||            ||

||                                  |        ||            ||

||-----------------------------------        ||            ||

||                                           ||            ||

|\-------------------------------------------/\------------/|

|/---------------------------------------------------------\|

||          information line                               ||

|\---------------------------------------------------------/|

|/-------------------------------------------\/------------\|

||                                           ||            ||

||                                           || Command    ||

||    Font selection area                    || selection  ||

||                                           || area       ||

||                                           ||            ||

|\-------------------------------------------/\------------/|

\-----------------------------------------------------------/

The  testcard editing area is subdivided into the visible  raster

and sync areas. The syncs are in two shades of grey and shown  to

confirm  the integrity of the sync pattern when a testcard  image

is  loaded for editing. You cannot change the sync part  of  this

area  although  it is possible to corrupt the syncs  by  changing

parts  of  the font. See the font editing  instruction  for  more

details. The Visible part of the raster shows the positioning  of

characters from the font or colours from the colour selection box

in the top right corner. To place a character, move the cursor to

the  character  or colour you require then press the  LEFT  mouse

button.  Move the cursor to the destination within the  card  and

press the RIGHT mouse button to "drop" it there.

(testcard editing area. cont.)

Once a character has been selected it can be dropped many  times,

there is no need to pick it up each time.

You need to be careful when changing from colour boxes to charac-

ters.  After the colour you must place a control character  (from

right  end  of colour bar) to force the generator  to  revert  to

character  mode.  This unfortunately means there must  be  a  gap

between any colour and text following it.

As  an  aid to aligning the characters, a dim grey  grid  can  be

overlayed by clicking the LEFT mouse button on the "enable  grid"

box  in  the command area. The grid will be redrawn each  time  a

character  is placed in the editing area and this takes  time  so

you may prefer to work without it.

The information line.

When preparing several similar testcards I kept forgetting  which

one  I was working on. To jog my memory I added  the  information

line  and displayed the name of the file being edited on  it.

At  the right end of the line the current position of the  cursor

and  character number are shown. The numbers are in  Hexadecimal,

if you need to work out real memory addresses this is more useful

than  decimal.  Character numbers range between 80H and  FFH  ie.

they all have bit 7 set. Colours work differently, the red, green

and blue are controlled by individual bits in the byte.

The colour selection box.

Pressing  the LEFT mouse button will select the colour under  the

cursor. White is available from the character font box.

A  control  (CTRL) character must be placed after the  colour  if

changing  to  text unless the colour is already at the end  of  a

line.

Colour character codes do not have bit 7 set.

The font selection area.

Pressing  the mouse LEFT button while the cursor is over  one  of

the  characters  in the font box will select that  character  for

placing in the testcard area or in the font edit box.

Any  character can be placed in the testcard but be very  careful

if  you transfer any of the first 16 characters to the font  edi-

tor.  These characters have a dual purpose, they display as  seen

in the font area but also control the shape of the sync waveform.

If  you  change any of these you risk producing a  poorly  locked

picture  or one running at the wrong vertical or horizontal  scan

frequency.

The font editing area.

This is where changes to any of the characters can be made. First

select  a  character from the font selection area then  move  the

cursor  to  the command area and click the LEFT button  on  "edit

font".  The chosen character will be redrawn in a  large  editing

box.  Each cell in the box represents one pixel in the  character

and  its  contents can be turned on or off by  moving  the  mouse

cursor to the cell and clicking the LEFT button. The button has a

toggle action, each click changes the state of that pixel from on

to off or vice versa.

To  leave the font editor and resume editing the  testcard,  move

the  cursor to either the "accept" or "reject" boxes in the  com-

mand area. Accepting the change will copy the new character  back

to the font and update the font selection display. If the  change

is rejected, the testcard edit resumes without updating the font.

As  mentioned earlier, the first 16 characters are also  used  to

produce the synchronising and equalising pulses so be very  care-

ful if you decide to change them.

Once a change has ben made and accepted the only way to "undo" it

is to edit the character back to its original condition.

The command selection area.

The  seven control functions are selected by placing  the  cursor

over the desired box and pressing the LEFT mouse button.

Only  those  options applicable to the current state  of  editing

will  have  any effect. These are highlighted to  make  selection

more obvious.

Enable  grid - makes the testcard alignment grid appear when  the

next character is placed there.

Disable grid - removes the alignment grid from the testcard  area

when the next character is placed there.

Edit  font - places the selected character in the font edit  area

and enables font edit mode.

Reject change - throws away the edit font area and resumes  test-

card editing.

Accept  change - copies the font edit area to the font  selection

area then updates the font and testcards selection areas.

Save & exit - processes the current font and testcard data into a

format suitable for programming to an EPROM the writes the infor-

mation back to disk.

Quit  - does just that, restores the original video mode,  throws

all  changes away and returns to DOS or whatever called  Cropedit

in the first place.

Mouse controls:

/-----------\

Pick character   |   |   |   |

|   |   |   |

or       --->|   |   |   |<---  Place character in  testcard

|   |   |   |

Toggle pixel     |   |   |   |

|-----------|

or           |           |

|           |

Select function  |           |

|           |         The middle mouse button

|           |         (if present) has no

|           |         effect.

|           |

|           |

|           |

\-----------/

Program invocation:

Start the program with CROPEDIT <image_name> <--/

where  <image_name> is the name of a testcard image. Samples  are

provided with this program to get you started.

When the "save & exit" command is selected, the new file is saved

under  the  same name as the original. I suggest you  create  and

save  templates to use for editing later and edit renamed  copies

of these, keeping the originals safe.

Errors:

The program tries to trap certain errors and returns a message to

help  you  find the problem. It returns errorlevel 0  to  DOS  if

everything  is  OK otherwise the errorlevel is set to  the  error

number in the message.

Err01 - mouse not present. (or mouse driver not responding)

Err02 - file name missing from command line

Err03 - the file could not be found or a disk error occured.

Whats what:

these files were released in this package,

CROPEDIT.COM - the executable program

CROPEDIT.DOC - you are reading it now

BLANK.TCG    - sample testcard, syncs but no video

CRHATCH.TCG  - sample testcard, crosshatch, bars, multi-burst

COLBARS.TCG  - sample testcard, colour bars and red bar

Please  distribute this package in its original .ZIP format  with

all these files present.

The  latest version of Cropedit is available from BetWiXt BBS  in

South  Wales.  The BBS specialises in programming  utilities  and

amateur  radio/TV  topics. Limited access on first call  but  up-

grades to full access are usualy made within 24 hours to  legiti-

mate callers.

BBS Details:

BetWiXt BBS.

available 24 hours a day

300 - 33,600 Baud V34 / V90 & V42 / MNP5

telephone  (international) +44 633 614 765

(from UK) 0633 614765

Please note that the number is subject to change. The latest news

of the BBS will be available on the BATC and Mustang software web

sites, "www.batc.org.uk" and "www.mustang.com" respectively.

SHAREWARE:

This program is not crippled in any way and has no expiry date.

It did take a long time to write and debug, and I make no guaran-

tee  that  all the bugs are ironed out. Let me know if  you  find

any.

BetWiXt  is a free BBS, I do not charge for its use but  it  does

cost  me to keep it running. Please try this program and  if  you

find it acceptable send a small donation towards the BBS  running

costs.  I suggest 5 UK pounds or 10 US dollars or its  equivalent

in  your currency, more than this is always welcome. If  you  are

outside  the UK, please do not send cheques (checks)  because  my

bank charges 25 UK pounds to process them, please send bank notes

instead and make sure they cannot be seen through the envelope.

If  you want to pay by cheque from the UK please make it  payable

to Mr B. E. Kelly.

The BBS postal address is:

BetWiXt BBS.

12 Cotswold Way,

Risca,

Gwent.  NP1 6QT

U.K.

I consider continued use of Cropedit without making a donation to

be unethical. Please support the shareware concept by  supporting

shareware authors.

I absolutely prohibit the sale of this program to any individual,

you may only charge a reasonable amount for media and duplicating

costs if distributing it as a shareware program.

If  you  wish  to include Cropedit as part  of  another  package,

please  contact  me for permission and include  details  of  that

package.

Brian Kelly.   GW6BWX.

March 7, 1993

Revision History:

v1.0 1989 Original program release.

v2.0 1992 speeded up grid drawing routines, fixed minor bug

causing  first bit in the font not to be rotated,

this had no overall effect as the same bit was never

rotated back again before saving the font.

v3.0 30 January 1993

speeded up grid drawing routine again, added command

highlighting, added x,y and character values to the

display, moved the screen areas around a little to

improve readability. Prevented the syncs being edited

in the testcard edit area.

v3.1 3 February 1993

rewritten using instructions present on all 80xx micro-

processors. Earlier versions worked on 286 or better

only, the docs warned it wouldn't work on 8086/8088

machines but nobody read them! To avoid the complaints

that machines locked up running Cropedit, I took the

286+ instructions out.

v3.2 4 February 1993

theres always one last bug !

Of little importance but the buffer Cropedit uses to

store the testcard while working on it was not being

cleared. This *may* have resulted in spurious data

being left in an unused area of the testcard.

v3.3 7 March 1993

minor change to grid drawing routine, no visible or

functional changes.

Documentation ammended 17 March 1998 to advise of BBS phone

number changes and web addresses.